CARBON PASSPORT

Mitigation & Decarbonization
Decision Engine Blueprint

Developer-ready product architecture for turning emissions data into prioritized, measurable, verifiable reduction actions

| MEASURE Live & reported emissions | MITIGATE Recommend & execute actions | VERIFY Prove actual reductions |
| --- | --- | --- |

North-star product principle Carbon Passport must never recommend a mitigation without explaining the trigger, estimated tCO₂e reduction, financial impact, confidence level, assumptions, evidence required and the method by which the realized reduction will be verified.

Prepared for Saurient AI Labs Pvt. Ltd.
Product, Engineering, MRV, Sustainability & Verification Teams
Version 1.0 | October 2026

# 1. Executive Summary

Carbon Passport should evolve from an emissions accounting and passport platform into a closed-loop decarbonization operating system. The mitigation layer should convert measured Scope 1, Scope 2, Scope 3, facility, process, supplier, batch and product-carbon-footprint data into ranked interventions that can be approved, implemented, measured and independently verified.

The target operating model MEASURE → DETECT HOTSPOT → DIAGNOSE → RECOMMEND → SIMULATE → PRIORITIZE → APPROVE → IMPLEMENT → MONITOR → VERIFY → UPDATE CCF/PCF/PASSPORT

## What the decision engine must accomplish

Identify the largest controllable emission hotspots at facility, process, asset, supplier and product level.

Match each hotspot with technically relevant mitigation measures from a controlled mitigation library.

Estimate reduction potential, cost, annual savings, payback, marginal abatement cost and implementation complexity.

Rank recommendations using transparent rules rather than opaque AI-only suggestions.

Convert accepted recommendations into mitigation projects with owners, milestones, evidence and MRV plans.

Compare baseline vs post-implementation performance and calculate realized reduction.

Route material reductions for verifier review and prevent double counting.

Propagate verified reductions into organizational footprints and product passports.

## Standards alignment

The design should align with GHG Protocol accounting boundaries and Scope 3 hotspot identification, ISO 14064 principles for quantification/reporting/verification, and the SBTi implementation hierarchy that prioritizes direct activity-level emissions reductions before broader shared-system or sector-level interventions. Carbon credits and removals must remain visibly separated from operational reductions.

| Reference | How Carbon Passport should use it |
| --- | --- |
| GHG Protocol | Inventory boundaries, Scope 1/2/3 categorization, emission factors, Scope 3 hotspot focus and transparent reporting. |
| ISO 14064-1 / 14064 series | Quality controls, quantification methods, documentation, inventory governance and verification readiness. |
| SBTi Corporate Net-Zero Standard V2.0 | Prioritize direct/activity-level mitigation first; use shared-system actions where relevant; clearly disclose assumptions and progress. |
| Internal MRV methodology | Project-specific baseline, normalization, monitoring period, evidence, verifier workflow and frozen verified results. |

# 2. Carbon Passport Mitigation Product Principles

| Principle | Required behavior |
| --- | --- |
| 1. Reduction before compensation | Operational efficiency, process changes, electrification, renewable energy and supplier transformation must be prioritized before offsets/credits. |
| 2. Explainable decisions | Every recommendation displays trigger rules, data used, assumptions, formulas and confidence. |
| 3. Measured beats estimated | SaurEdge/interval-meter/production telemetry should upgrade confidence and verification quality whenever available. |
| 4. Financial + carbon view | The decision engine optimizes both tCO₂e and business value; managers need CAPEX, OPEX, savings and payback. |
| 5. Baseline integrity | No “verified reduction” without a documented baseline, boundary, normalization method and monitoring period. |
| 6. No double counting | A reduction can be attributed once within the defined reporting boundary; downstream passport propagation must preserve lineage. |
| 7. Human approval | AI may recommend and summarize; material project approval, engineering feasibility and verifier sign-off remain human-controlled. |
| 8. Version everything | Emission factors, recommendation rules, baselines, scenarios and verified outputs require version history and audit trails. |

# 3. Decision Engine Architecture

Recommended architecture Data Ingestion → Emission Normalization → Hotspot Engine → Mitigation Matcher → Scenario Engine → Financial Engine → Priority Scoring → Recommendation UI → Project Workflow → MRV Engine → Verification → Passport Update

| Layer | Inputs | Primary output |
| --- | --- | --- |
| 1. Data ingestion | Bills, SaurEdge telemetry, meters, fuel, ERP, BOM, logistics, supplier declarations | Normalized activity data |
| 2. Emission calculation | Activity × factor, PCF/CCF rules | tCO₂e by source/time/product |
| 3. Hotspot detection | Contribution %, intensity, trends, anomalies | Ranked emission hotspots |
| 4. Diagnostic logic | Equipment metadata, operating hours, baselines, peer thresholds | Likely causes / opportunities |
| 5. Mitigation matcher | Hotspot taxonomy + eligibility rules | Candidate mitigations |
| 6. Scenario engine | Technical assumptions + user inputs | Expected reduction ranges |
| 7. Financial engine | CAPEX/OPEX/tariff/savings/incentives | ROI, payback, NPV-ready metrics |
| 8. Priority engine | Carbon, economics, confidence, ease, strategic fit | Priority score & recommended sequence |
| 9. Project workflow | Approval, owner, budget, vendor, timeline | Mitigation project |
| 10. MRV & verification | Baseline + telemetry + evidence | Realized and verified reduction |
| 11. Passport propagation | Verified project allocation | Updated CCF/PCF/passport claims |

# 4. Minimum Data Model for Mitigation

| Entity | Key fields required |
| --- | --- |
| EmissionSource | source_id, scope, category, facility, asset/process, fuel/energy type, activity, unit, factor_id, emissions_tco2e, time period |
| Hotspot | hotspot_id, source_id(s), contribution_pct, intensity, trend, anomaly_score, controllability, diagnostic tags |
| MitigationMeasure | measure_id, category, eligible_source_types, trigger rules, contraindications, calculation method, evidence requirements |
| Scenario | scenario_id, hotspot_id, measure_id, baseline, assumptions, reduction_low/base/high, CAPEX, OPEX_delta, annual_saving, payback, confidence |
| MitigationProject | project_id, approved scenario, owner, facility, status, dates, vendor, budget, target_reduction, MRV_plan |
| Baseline | baseline_id, boundary, period, raw data references, adjustment variables, calculation_version, locked_hash |
| MRVRecord | project_id, monitoring_period, actual_data, normalized_counterfactual, reduction_tco2e, uncertainty, evidence |
| Verification | verification_id, verifier, status, findings, approved_reduction, decision_date, signature/hash |
| ReductionAllocation | verified_reduction_id, organization/facility/process/product/batch allocation, period, anti-double-counting keys |

# 5. Decision Engine Logic - From Emission to Action

## 5.1 Step 1 - Determine whether the source is material

| Rule | Example threshold | Decision |
| --- | --- | --- |
| Absolute contribution | ≥5% of facility emissions | Material hotspot |
| High intensity | >benchmark or internal target by ≥10% | Efficiency opportunity |
| Rapid increase | >10% vs normalized prior period | Investigate anomaly |
| High controllability | Owned/operated asset or contracted activity | Prioritize action |
| Regulatory/strategic relevance | CBAM/product/customer/target exposure | Raise priority even below materiality threshold |

## 5.2 Step 2 - Diagnose the probable driver

Load/usage growth

Operating-hours growth

Low equipment efficiency

Idle/standby consumption

Leaks/losses

Fuel mix

Grid factor or procurement mix

Poor control/set-point

Maintenance condition

Production mix change

Supplier/material intensity

Transport mode/distance/load factor

Waste generation or low recovery

## 5.3 Step 3 - Generate only eligible mitigations

Eligibility rule A recommendation should be generated only if minimum input requirements are met. If required engineering data is missing, show “Assessment Required” rather than pretending to know the saving.

## 5.4 Step 4 - Calculate a reduction range, not a false precise value

Use Low / Base / High estimates. The base case may be deterministic where telemetry and engineering parameters are available; otherwise it should be an assumption-backed estimate. Always expose the formula.

| Calculation type | Illustrative formula |
| --- | --- |
| Efficiency improvement | Avoided energy = Baseline normalized consumption × expected efficiency improvement % |
| Renewable generation | Avoided grid electricity = eligible on-site renewable generation × applicable factor, subject to accounting rules |
| Fuel switch | Reduction = baseline fuel emissions − projected replacement energy/fuel emissions |
| Transport mode | Reduction = tonne-km × (baseline mode EF − new mode EF) |
| Material substitution | Reduction = quantity × (baseline material EF − substitute EF), adjusted for yield/performance |
| Supplier improvement | Reduction = purchased quantity × verified supplier PCF improvement |

# 6. Recommendation Priority Scoring

The first production version should use transparent weighted scoring. Later, machine learning can improve diagnostics but should not replace explainable ranking.

| Dimension | Weight | Score guidance (0-100) |
| --- | --- | --- |
| Carbon reduction potential | 30% | Higher annual tCO₂e reduction and % of hotspot = higher score |
| Financial attractiveness | 20% | Shorter payback / stronger savings = higher score |
| Confidence / data quality | 15% | Direct telemetry + stable baseline > invoices > estimates |
| Implementation feasibility | 15% | Low downtime, available technology/vendor, manageable approvals |
| Strategic/regulatory relevance | 10% | Target, CBAM, customer, product-passport or risk relevance |
| Speed to impact | 10% | Faster measurable impact = higher score |

Suggested formula Priority Score = 0.30(Carbon) + 0.20(Financial) + 0.15(Confidence) + 0.15(Feasibility) + 0.10(Strategic) + 0.10(Speed). Also apply “gates”: safety, legal, technical infeasibility and insufficient-data flags can suppress or downgrade a recommendation.

| Priority band | Score | UI treatment |
| --- | --- | --- |
| P1 - Immediate | 80-100 | Top recommendation; create project CTA |
| P2 - High | 65-79 | Strong candidate; feasibility review |
| P3 - Medium | 45-64 | Scenario/assessment required |
| P4 - Low | <45 | Backlog / monitor |
| Blocked | Any | Safety, technical, data or policy gate failed |

# 7. Mitigation Library - What Carbon Passport Should Showcase

## 7.1 Industrial - Electricity / Scope 2

| Mitigation | Trigger examples | Required inputs | MRV method |
| --- | --- | --- | --- |
| After-hours load reduction | Night baseload >20-30% of day load | Interval kW/kWh, operating schedule | Pre/post interval model normalized for operating hours |
| Compressed-air optimization | High compressor run time, abnormal baseload, leak survey flag | kWh, pressure, flow if available, production | kWh per production unit + leak repair evidence |
| VFD / motor optimization | Variable-load motor operated fixed-speed | motor kW, load profile, hours | submeter or equipment kWh before/after |
| HVAC/chiller optimization | High cooling share, poor kW/TR or excessive runtime | chiller kWh, temperature, runtime, weather | weather-normalized kWh and cooling output |
| LED / lighting controls | High lighting hours or legacy fixtures | fixture count, wattage, hours | engineering calc + circuit/whole-building validation |
| Power factor correction | PF below configured threshold | kW, kVA, PF, tariff rules | metered PF and demand/penalty effect |
| Rooftop solar | Suitable load/roof/resource and grid eligibility | load, roof, irradiance estimate, tariff | inverter generation + import meter + accounting method |
| Renewable procurement | Residual Scope 2 remains material | consumption, market instrument eligibility | certificate/contract retirement + accounting controls |

## 7.2 Industrial - Scope 1 / Process

| Mitigation | Typical source | Examples |
| --- | --- | --- |
| Boiler/burner optimization | Natural gas/LPG/diesel/biomass combustion | O2 tuning, blowdown optimization, condensate return, insulation |
| Waste heat recovery | Furnaces, kilns, boilers, compressors | pre-heating, hot-water recovery, ORC where appropriate |
| Electrification | Fuel-fired process or material handling | electric boiler/heat pump where feasible, electric forklift |
| Fuel switching | High-carbon fuel | lower-carbon fuel subject to lifecycle and technical assessment |
| Refrigerant management | HVAC/cold storage | leak detection, low-GWP refrigerant transition, recovery records |
| Fleet decarbonization | Owned vehicles | EV transition, route optimization, idle reduction |

## 7.3 Scope 3 / Product & Supply Chain

| Mitigation | Decision-engine signal | Passport value |
| --- | --- | --- |
| Supplier substitution/engagement | Supplier PCF materially above category benchmark | Lower upstream PCF with supplier evidence |
| Material substitution | Material dominates product footprint | Scenario PCF comparing alternate materials |
| Recycled content | Virgin input emission intensity high | Track recycled % and supplier evidence |
| Logistics optimization | Air/road freight dominates | Mode, route, load-factor scenarios |
| Packaging redesign | Packaging intensity or waste high | Reduced material weight and PCF |
| Yield/scrap improvement | High scrap/rework rate | Reduced input per saleable unit |
| Circular/reuse model | Product/material suitable for recovery | Avoided virgin input; report carefully and separately from inventory claims |

## 7.4 Corporate Offices

| Area | Recommendations Carbon Passport can surface |
| --- | --- |
| HVAC | Set-point optimization, scheduling, zoning, preventive maintenance, chiller efficiency, fresh-air control |
| Lighting | LED, daylight control, occupancy sensors, after-hours shutdown |
| Plug loads | PC/laptop power policies, pantry equipment schedules, smart plugs, printer consolidation |
| UPS / server rooms | UPS loading/efficiency, cooling optimization, virtualization/cloud workload review |
| Pumps / water | Pump scheduling, VFDs, leak detection |
| Renewables | Rooftop solar or eligible renewable procurement |
| Employee travel | Business-travel policy, rail substitution, virtual meeting policy, EV commuting programs |
| Waste | Segregation, food-waste reduction, recycling, vendor evidence |

# 8. Worked Examples for the Decision Engine

## Example A - Corporate office overnight baseload

| Field | Example |
| --- | --- |
| Detected condition | Average 10 PM-5 AM load = 24 kW; occupied-hours average = 70 kW |
| Trigger | Overnight baseload = 34% of occupied-hours average; threshold configured at 25% |
| Diagnosis candidates | HVAC left on, UPS/server load, lighting, pantry loads, pumps, signage |
| Recommendation | Run 7-day circuit-level audit; implement shutdown schedules for controllable loads |
| Estimated saving | Low 6%, Base 10%, High 14% of relevant electricity segment |
| Evidence required | Operating schedule, BMS/controls settings, before/after interval data |
| Verification | Compare normalized after-hours kWh before vs after; exclude approved 24x7 loads |
| Decision output | P1 if payback <12 months and data confidence ≥80 |

## Example B - Factory compressed-air system

| Field | Example |
| --- | --- |
| Detected condition | Compressor consumes 18% of plant electricity and maintains high load during non-production hours |
| Trigger | Share >10% + idle consumption >configured threshold |
| Recommendation stack | Leak survey → pressure optimization → sequence control → VFD/efficient compressor assessment |
| Scenario | Base-case reduction = 12% of compressor electricity |
| Financial output | Annual kWh saved × tariff; CAPEX from user/vendor estimate; simple payback displayed |
| MRV | Compressor kWh normalized by production output, operating hours and pressure set-point |
| Verification | Maintenance evidence + telemetry + reviewer approval |
| Passport effect | Allocate verified process-energy reduction to affected product batches by defined allocation rule |

## Example C - Rooftop solar

| Field | Example |
| --- | --- |
| Hotspot | Scope 2 = 68% of organization footprint |
| Eligibility | Daytime load exists; roof/land feasibility unknown |
| Engine behavior | Do not claim exact reduction until system size and generation estimate are provided |
| Recommendation | Solar feasibility assessment with 100/250/500 kWp scenarios |
| Scenario outputs | Annual generation, self-consumption, exported electricity, avoided grid electricity, CAPEX, savings, payback |
| MRV | Inverter + import/export meter + monthly reconciliation |
| Accounting control | Apply current Scope 2 accounting rules; preserve instrument/source evidence and prevent double counting |

## Example D - Product-level material change

| Field | Example |
| --- | --- |
| Product | Steel component, 1,000 kg batch |
| Hotspot | Purchased steel contributes 62% of PCF |
| Recommendation | Evaluate lower-emission steel supplier or higher recycled-content grade |
| Calculation | Quantity × supplier-specific PCF difference, adjusted for yield and specification |
| Guardrail | Engineering/specification approval required; do not assume functional equivalence |
| Verification | Supplier declaration/EPD/PCF evidence + procurement records |
| Passport output | Show previous PCF, new PCF, reduction %, mitigation action and verification status |

# 9. Confidence & Data Quality Model

| Confidence tier | Typical evidence | How UI should label it |
| --- | --- | --- |
| A - Verified telemetry | Calibrated meter/SaurEdge + locked baseline + verified implementation evidence | High confidence |
| B - Direct measured | Interval meter or direct utility/fuel data with adequate baseline | Measured estimate |
| C - Engineering estimate | Equipment specifications, operating hours and accepted engineering assumptions | Engineering estimate |
| D - Spend/benchmark proxy | Spend-based, generic benchmark or incomplete asset data | Screening estimate only |

Important guardrail A low-confidence recommendation may still be useful, but the engine must recommend the next data-collection action needed to improve confidence—for example “Install submeter”, “Capture compressor pressure/flow”, or “Request supplier PCF”.

# 10. MRV and Verification Workflow

| Stage | System behavior | Required evidence |
| --- | --- | --- |
| 1. Recommendation | Create scenario; no reduction claim yet | Data + assumptions |
| 2. Approved | Freeze approved scenario/version | Approval, budget, owner |
| 3. Implemented | Record commissioning date | Invoice/work order/photos/specs |
| 4. Monitoring | Collect post-project data | Telemetry/bills/production/weather as applicable |
| 5. Calculated | Compute normalized counterfactual and realized saving | Baseline model + monitoring dataset |
| 6. Under verification | Freeze record; verifier reviews | Complete evidence pack |
| 7. Verified | Verifier approves reduction amount/period | Verification statement/findings |
| 8. Allocated | Apply reduction to reporting/product boundaries | Allocation rule + anti-double-counting key |
| 9. Passport update | New PCF/CCF version issued where applicable | Version lineage + hash |

## Baseline rules

Baseline period must be representative; flag shutdowns, abnormal production or temporary events.

Normalize for relevant drivers such as production volume, weather, occupancy, operating hours, product mix or floor area.

Never compare raw “before” and “after” totals when activity level materially changed.

Lock baseline version when the project is approved; revisions require reason, authorization and audit history.

Report uncertainty and data gaps; do not convert screening estimates directly into verified reductions.

# 11. Recommended Product Screens

| Screen | Must show |
| --- | --- |
| Mitigation Overview | Baseline emissions, current emissions, target gap, identified potential, active projects, verified reductions, cost savings |
| Hotspot Explorer | Scope/category/facility/process/product drill-down, contribution, intensity, trend, controllability |
| Recommendations | Rank, reason, reduction range, CAPEX, savings, payback, confidence, difficulty, create-project action |
| Recommendation Detail | Trigger, source data, assumptions, equations, prerequisites, risks, evidence, scenario controls |
| Scenario Builder | Low/base/high assumptions; energy/fuel/material/tariff inputs; sensitivity; compare scenarios |
| Mitigation Project | Owner, milestones, vendor, budget, evidence, MRV plan, status |
| MRV Dashboard | Baseline vs expected vs actual, normalized metrics, telemetry trend, variance, realized tCO₂e |
| Verifier Workspace | Evidence pack, calculation trace, findings, approve/reject/request changes |
| Verified Reductions Registry | Unique reduction records, period, boundary, allocation, verifier, status, anti-double-counting identifiers |
| Passport Mitigation Tab | Actions affecting the product, previous/current PCF, reduction %, evidence and verification status |

# 12. Recommendation Card - Required Output Contract

UI example COMPRESSOR OPTIMIZATION ¦ Priority P1 Why flagged: Compressor electricity = 18% of plant load; non-production baseload is 42 kW. Potential reduction: 24-41 tCO₂e/year (base: 32) Estimated annual saving: ₹4.6 lakh ¦ CAPEX: ₹1.4 lakh ¦ Payback: 3.7 months Confidence: 82% (Measured) ¦ Complexity: Low-Medium Next step: Perform leak survey + pressure/sequence assessment Verification: Compressor kWh per tonne of production, 30-day pre/post normalized comparison

| API/UI field | Mandatory? | Notes |
| --- | --- | --- |
| recommendation_id | Yes | Stable identifier |
| title/category | Yes | Human-readable action |
| hotspot_ids | Yes | Traceability to emission sources |
| trigger_explanation | Yes | Plain-language reason |
| reduction_low/base/high_tco2e | Yes | No single-point-only estimate |
| baseline_value/unit | Yes | Calculation context |
| capex/opex/savings/payback | If available | Clearly mark user/vendor/benchmark assumptions |
| confidence_score/tier | Yes | Expose data quality |
| priority_score/band | Yes | Include scoring breakdown |
| assumptions | Yes | Editable/versioned |
| prerequisites/risks | Yes | Safety/engineering/policy gates |
| evidence_requirements | Yes | For implementation and verification |
| mrv_method | Yes | How actual reduction will be measured |
| status | Yes | Recommended/approved/implemented/etc. |

# 13. Starter Rules for V1 Decision Engine

| Rule ID | IF condition | THEN action | Guardrail |
| --- | --- | --- | --- |
| ELEC-001 | After-hours baseload >25% of occupied average for ≥5 days | Recommend after-hours load audit | Exclude declared 24x7 critical loads |
| ELEC-002 | Power factor <0.90 for configured share of intervals | Assess PF correction | Financial claim depends on tariff/demand structure |
| ELEC-003 | Electricity >40% of footprint and daytime load material | Run solar/renewable scenario | Requires feasibility and accounting eligibility |
| HVAC-001 | Cooling electricity intensity >benchmark or rises >10% normalized | HVAC/chiller optimization assessment | Normalize for weather/occupancy |
| AIR-001 | Compressor non-production load >20% of production-period average | Leak/pressure/sequence optimization | Need schedule and compressor-meter confidence |
| FUEL-001 | Stationary combustion >15% of footprint | Boiler/burner/heat-recovery assessment | Engineering/safety review required |
| REF-001 | Refrigerant emissions material or leak events detected | Leak management / refrigerant transition | Use refrigerant mass-balance records |
| LOG-001 | Air freight >configured % of Scope 3 transport | Mode-shift scenario | Check service/lead-time constraints |
| MAT-001 | One material >30% of PCF | Supplier/material-substitution scenario | Functional/spec equivalence required |
| SUP-001 | Supplier PCF >category benchmark by >20% | Engage/substitute supplier | Benchmark must be versioned and comparable |
| WASTE-001 | Scrap rate exceeds target by >X% | Yield/scrap-reduction project | Normalize for product mix |
| DATA-001 | Hotspot material but data confidence <50% | Recommend data-improvement action first | No investment recommendation until minimum data gate |

# 14. Developer Implementation Checklist

## 14.1 Foundation & Data

☐  Define canonical emission-source taxonomy across Scope 1/2/3, facility, asset, process, supplier, product and batch.

☐  Create versioned emission-factor registry with geography, year, source and unit conversions.

☐  Normalize all activity data to canonical units and retain original source/value.

☐  Implement source-to-emission lineage and immutable calculation version IDs.

☐  Add asset/equipment metadata required by mitigation rules.

☐  Add production/occupancy/weather/operating-hours adjustment variables.

☐  Implement telemetry quality checks: gaps, flatlines, outliers, clock drift and missing intervals.

## 14.2 Hotspot Engine

☐  Calculate absolute tCO₂e, share %, intensity and trend for every hierarchy level.

☐  Allow configurable materiality thresholds by customer/sector.

☐  Calculate controllability and strategic relevance tags.

☐  Detect abnormal increases on normalized data.

☐  Provide a plain-language “why this is a hotspot” explanation.

## 14.3 Mitigation Library

☐  Create MitigationMeasure master table and taxonomy.

☐  Add eligibility criteria, exclusions, prerequisites and safety/engineering gates.

☐  Add calculation templates for energy efficiency, fuel switch, renewable, logistics and material measures.

☐  Add evidence checklist and MRV method for every measure.

☐  Add geography/sector applicability and implementation complexity.

☐  Version mitigation rules and assumptions.

## 14.4 Scenario & Financial Engine

☐  Support Low/Base/High reduction scenarios.

☐  Allow user override with justification and audit history.

☐  Calculate annual tCO₂e reduction, kWh/fuel/material saving, CAPEX, OPEX change, annual saving and simple payback.

☐  Calculate marginal abatement cost (currency/tCO₂e) where inputs allow.

☐  Add sensitivity variables such as tariff, production growth, emission factor and equipment life.

☐  Separate estimated savings from verified realized savings.

## 14.5 Priority Engine

☐  Implement configurable weighted priority scoring.

☐  Expose scoring breakdown in UI/API.

☐  Implement hard gates for insufficient data, safety, technical incompatibility and policy/accounting conflicts.

☐  Prevent “AI confidence” from overriding data-quality confidence.

☐  Allow enterprise administrators to tune weights without modifying code.

## 14.6 Project Workflow

☐  Convert approved recommendation into project without retyping data.

☐  Assign owner, target, budget, dates, vendor, milestones and site.

☐  Lock approved scenario/version as project target.

☐  Create evidence upload checklist and status tracking.

☐  Generate reminders for missing monitoring data or milestones.

## 14.7 MRV & Verification

☐  Create baseline object with boundary, period, normalization variables and hash.

☐  Implement pre/post and counterfactual calculation methods.

☐  Calculate realized reduction and uncertainty/confidence.

☐  Freeze records when submitted for verification.

☐  Provide verifier approve/reject/request-change workflow.

☐  Create immutable verified-reduction record with unique identifier.

☐  Prevent duplicate allocation of the same reduction across overlapping claims.

## 14.8 Passport & Reporting

☐  Link verified reductions to affected CCF/PCF calculation versions.

☐  Show previous and current footprint with change attribution.

☐  Show reduction actions separately from offsets/removals/credits.

☐  Include evidence/verification status in passport view without exposing confidential documents publicly.

☐  Keep full audit trail for every change.

# 15. Minimum Acceptance Criteria for V1

| Capability | V1 acceptance test |
| --- | --- |
| Hotspot detection | Given sample facility data, engine ranks top emission sources and explains why. |
| Rule matching | At least 20 curated mitigation rules work across office + industrial electricity/fuel. |
| Scenario calculation | Every matched measure produces low/base/high reduction with assumptions. |
| Financial output | CAPEX/savings/payback available where inputs exist; unknowns are explicitly marked. |
| Explainability | User can view trigger rule, source data, equation and confidence. |
| Project conversion | Recommendation becomes project with locked target and evidence checklist. |
| MRV | One telemetry-based and one invoice/engineering-based project can calculate realized reduction. |
| Verification | Verifier can approve/reject and approved reduction is frozen. |
| Anti-double-counting | System rejects duplicate allocation of same verified-reduction record. |
| Passport update | Verified reduction can update a product/facility footprint with version lineage. |
| Audit export | Export shows source data, assumptions, formulas, project evidence index, verification and allocation. |

# 16. Suggested Product Roadmap

| Phase | Scope | Outcome |
| --- | --- | --- |
| Phase 1 - Rules & scenarios | Hotspots, 20-30 curated measures, scoring, scenario/financial model | Actionable recommendations |
| Phase 2 - Projects & MRV | Project workflow, baselines, SaurEdge/interval data, realized savings | Closed-loop mitigation |
| Phase 3 - Verification & passport | Verifier workflow, verified-reduction registry, PCF/CCF propagation | Auditable verified reductions |
| Phase 4 - AI advisor | Natural-language explanations, root-cause suggestions, scenario copilot | Faster user decision-making |
| Phase 5 - Ecosystem | Vendor/ESCO/solar/engineering partner marketplace, APIs, sector packs | Execution marketplace and scale |

# 17. AI Mitigation Advisor - Safe Product Boundary

| AI should | AI should NOT |
| --- | --- |
| Explain hotspots and rules in simple language | Invent equipment specs, prices or savings |
| Suggest candidate investigations/actions | Approve engineering/safety-critical changes |
| Summarize scenarios and trade-offs | Label an estimated saving as verified |
| Draft mitigation plans and evidence requests | Change locked baselines or verified records silently |
| Detect missing data and recommend next measurements | Hide uncertainty or assumptions |
| Help compare options | Count offsets as operational reductions |

# 18. Final Carbon Passport Mitigation Operating Model

The product promise Carbon Passport should answer five questions for every material emission: (1) Where did it come from? (2) Why is it high? (3) What can we realistically do about it? (4) What will the action cost and save? (5) Can we prove the reduction actually happened?

| Stage | User question | Carbon Passport response |
| --- | --- | --- |
| Measure | What are we emitting? | Scopes, PCF/CCF, telemetry, evidence |
| Understand | Where is the problem? | Hotspots, intensity, anomalies, drivers |
| Decide | What should we do first? | Ranked mitigations + scenarios + economics |
| Execute | How do we manage the action? | Project workflow, owner, vendor, milestones |
| Verify | Did it really reduce emissions? | MRV, normalized baseline, verifier approval |
| Prove | How do we communicate it? | Verified reduction registry + updated Carbon Passport |

MEASURE → UNDERSTAND → REDUCE → VERIFY → PASSPORT

# 19. Reference Frameworks

GHG Protocol - Standards & Guidance, including Corporate Value Chain (Scope 3) Standard and Scope 2 Guidance. https://ghgprotocol.org/standards-guidance

Science Based Targets initiative - Corporate Net-Zero Standard V2.0, Target Implementation hierarchy (2026). https://standards.sciencebasedtargets.org/chapter/4-target-implementation

ISO 14064-1:2018 - Greenhouse gases - Part 1: Specification with guidance at the organization level for quantification and reporting of greenhouse gas emissions and removals (confirmed current in 2024). https://www.iso.org/standard/66453.html

ISO/TS 14064-4:2025 - Guidance for the application of ISO 14064-1. https://www.iso.org/standard/86109.html

Note: The numerical thresholds and savings percentages in this blueprint are illustrative defaults for product design. Production deployment should allow sector/site configuration and should not present an engineering estimate as a verified result.